# Smart Algae Panel System

Project: Analyzing the Effects of Smart Algae (Chlorophyta) Panels for Thermal Heat Index Reduction

# 1. System Overview

This system measures:
 • CO2 concentration
 • Temperature
 • Humidity
 • Raw pH sensor signal

 The Arduino collects RAW data only. All analysis must be done externally.

# 2. Install Arduino IDE

1. Go to this link Arduino or click this direct link.
 2. Download Arduino IDE
 3. Install using default settings
 4. Launch Arduino IDE after installation

# 3. Connect the Arduino

1. Plug Arduino Uno into computer using USB cable
 2. Open Arduino IDE
 3. Go to Tools → Board → Arduino Uno
 4. Go to Tools → Port → Select the COM port that appears

# 4. Install Required Libraries

In Arduino IDE:
 1. Click Sketch → Include Library → Manage Libraries
 2. Search and install:
    • SparkFun SCD4x Arduino Library
    • LiquidCrystal I2C by Frank de Brabander

# 5. Wiring Instructions

SCD40 Sensor:
 VIN → Arduino 3.3V
 GND → Arduino GND
 SDA → Arduino A4
 SCL → Arduino A5

 I2C LCD:
 VCC → Arduino 5V
 GND → Arduino GND
 SDA → Arduino A4
 SCL → Arduino A5

 pH Sensor Module:
 V+ → Arduino 5V
 G → Arduino GND
 Po → Arduino A0



Smart Algae Panel Wiring Diagram shows the complete physical connections between the Arduino Uno, SCD40 sensor, I2C LCD, and pH module.

# 6. Upload the Firmware

1. Open the Smart Algae firmware (.ino file) (Go to File → Open→ SmartAlgaePanel.ino)
 2. Click Verify (✓)
 3. Wait for 'Done compiling'
 4. Click Upload (→)
 5. Wait for 'Done uploading'

# 7. First Startup

After upload:
 • LCD shows splash screen
 • Title scroll appears
 • Sensor initializes
 • Live readings begin

# 8. Viewing Data

To view readings on computer:
 1. Open Serial Monitor
 2. Set baud rate to 9600
 3. Observe data lines for CO2, temperature, humidity, and pH RAW

# 9. Proper Handling of Sensors

SCD40:
 • Do not cover holes
 • Avoid water splashes

 pH Probe:
 • Fragile glass tip
 • Do not drop
 • Keep slightly moist if stored

 Wiring:
 • Do not unplug while powered

# 10. Troubleshooting

LCD blank:
 • Check wiring
 • Adjust LCD contrast screw

 Upload fails:
 • Check correct COM port
 • Check USB cable

 Sensor error:
 • Restart Arduino
 • Check sensor wiring

# 11. Shutdown Procedure

1. Close Serial Monitor
 2. Unplug USB cable safely
 3. Store sensors properly